УДК: 53.043, 533

PACS: 47.10.ab

DOI:

Full name: Vadym Alexievich Ostanin

Self employed

Effects of repulsion and attraction between spinning discs in fluids

V.A. Ostanin

03039, 16 Demiivska Str, Kyiv, Ukraine

Abstract

Purpose. The effects of controllable attraction and repulsion of spinning circular object in a fluid is proposed. This study provides an overview of the push/pull effects caused by pair of spinning discs. The purpose of this study is to explain principles that cause push/pull effects pair of spinning circular objects in the fluid medium and investigate dependencies of force on relative spin directions and rotation speed. This study was inspired by magnetic effects caused by "spin" property of electrons and push/pull effects between two current-carrying wires.

Methods. The practical experiment uses a pair of plastic tubes that driven by motors and spin close to each other in air at an atmospheric pressure.

Results. A spinning disc adjacent to another spinning disc forms space of high or low pressure of air between them depending on spin direction. Spinning discs with opposite spin directions speed up air flow in gap that reduce air pressure between spinning objects and attracts them. Vise-versa, spinning discs with same directions damp air flow in gap that increase air pressure between spinning objects that repel them. Dependency between directions of spinning objects and observed ​effects had been verified by numerical simulation.

Conclusions. Observed repulsion and attraction effects can be exploited to manipulate objects in cases where magnetic or other touchless interactions aren’t possible. Also, it can be used as analogy to magnetic repulsion and attraction effects between two current-carrying wires.

Introduction.

Цель этой статья - описать и объяснить эффект контролируемого притяжение и отталкивания в несжимаемой флюидной среде основанный на законе Бернулли для газа или жидкости, если принять их как несжимаемые. Этот эффект пригодиться для инженерной задачи бесконтактного взаимодействия между объектами, но когда электрические или магнитные поля могут негативно повлиять на устройство. Описываемый эффект был обнаружен под вдохновением от изучения магнитных эффектов вызванных “спин” свойством электрона в проводниках с током. Но, как будет показано в статье, похожие эффекты наблюдаются и в флюидных средах. Закон Бернулли хорошо объясняет эффекты взаимодействия между объектами и потоками флюидов, как эффект Магнуса [1], и также, как будет показано в статье, отталкивания в зависимости от направления вращения пары и большего количества дисков или объектов круглой формы. Объектом исследования являются два пластиковых цилиндра длиной 10 см и диаметром 5 см, приводимые во вращение двигателями. Предметом исследования являются эффекты притяжения и отталкивания, возникаемые между вращающимися дисками в турбулентно воздушной среде при атмосферном давлении с числом Рейнольдса от ??? до 1.25×105.

A boundary layer will form on a rotating body of revolution due to the no-slip condition at the body surface as illustrated in Figure 6.2. At low values of the rotational Reynolds number, the flow will be laminar. As the rotational Reynolds number rises, the flow regime will become transitional and then turbulent [2].

Figure 1. Boundary layer flow over a rotating disc [2].

Re⌀=ρΩR2μ

Pressure differential

Список литературы

- Бутиков Е.И. Физика Кн.1 страницы 348 349

- Peter R.N. Childs. Rotating Flow, DOI: 10.1016/B978-0-12-382098-3.00006-8, Chapter 6